Lab 3 Bonus Problem: Barrel Shifter

A barrel shifter is a circuit that can shift or rotate a binary word or nibble by any number of bit positions in a single operation. Unlike a basic shift register, it does not shift by one operation per clock cycle, it shifts all bits combinationally at once

For a 4-bit input and 2-bit select, a barrel shifter can shift all bits left of right by 0 to 3 positions.

Either create your own multiplexers or add the two files under module 7 to your project. Build the Project as shown in diagram and fill in the table below. For the select K, describe the intermediate output P and output Y in terms of A.

| K1 K0 | P3 P2 P1 P0 | Y3 Y2 Y1 Y0 | Operation |
| --- | --- | --- | --- |
| 0 0 | A3 A2 A1 A0 | A3 A2 A1 A0 | No shift |
| 0 1 |  |  |  |
| 1 0 |  |  |  |
| 1 1 |  |  |  |

Adding a File to Your Project

Download the mux2.bdf and mux2.bsf files from Brightspace and add them to the project folder.

With the Quartus project open select file>open and select the mux2.bdf, make sure “add file to project” is checked. Now in the symbol menu, under project, you can select the mux2.bsf.

The provided mux2 consists of 5 inputs, your data inputs in1 and in2, the select K, an asynchronous clear clr, and the clock clk. Connect all sel0 to the K of the first set of mux2 and sel1 to the K inputs of the second set of mux2. Connect clear to the clr off al mux2.